Common Rafter Calculation

- Determine the Run of the rafter, which is ½ the span (the distance from outside wall to outside wall). Example: We are building a house that is 36’ x 40’. The roof is spanning the 36’ area of the house. So, our span is 36’ 0” and our run is 18’ 0”.

- Determine the slope of the roof, which is given on the elevation drawing in the house plans. For this example, we will use the slope of 5/12. The slope is given in the format of Rise/Run. This means that we have a 5” rise for every one foot of Run.

- Next we need to look at a rafter (framing) square to determine the multiplier. The multiplier for a 5/12 slope is 13”. You can also calculate the multiplier using the Pythagorean Theorem. Below I have provided a visual illustration of the triangle created by this slope.

13

5

12

- The next step is to calculate the line length of the common rafter: Run x Multiplier = line length.

So, 18’ x 13” = 234” or 19’ 6”

- Last, you need to subtract ½” the thickness of the ridge pole, which is 1 ½” thick. 1.5” x ½ = ¾”.     234” – ¾” = 233 ¼” or 19’ 5 ¼”

- This is the line length of the common rafter, which is from the center of the building to the outside walls. You will still have to calculate the overhang.