Touch a quantity for more information.

1.; Solving for Eb. 2.; Solving for Ee. 3.; Solving for Ie.

CALCULATING Av - Remember to use this formula:
  Av = Rc ÷ (.025 ÷ Ie)
  Rc is the collector resistor.
  Ie is emitter current and is calculated by following the steps above.

CALCULATING Eb - Since R1 and R2 form a voltage divider, Eb (Er2) is is found by:
  1. Finding Rt:  Rt = R1 + R2
  2. Solving for It:  It = Ea ÷ Rt 
  3. Calculating Eb:  Er2 = It x R2

CALCULATING Ee - Since the emitter to base PN junction drops 0.6 V, Ee equals:  Ee = Eb - 0.6 V

CALCULATING Ie - Ohm's Law is used to find emitter current (Ie):
  Ie = Ee ÷ Re